# THE DEVELOPMENT OF A MOBILE APPLICATION FOR MONITORING POULTRY EGG TEMPERATURE AND SECURITY UPDATES

## CHAPTER 1: INTRODUCTION

### 1.1 Background of the Study

#### 1.1.1 Overview of Poultry Production and Biological Constraints of Egg Storage

Poultry production stands as a cornerstone of global agriculture, providing an essential source of low-cost, high-quality animal protein in the form of meat and eggs to feed an expanding global population (Okubanjo, 2025). Within this sector, commercial poultry hatcheries rely heavily on achieving a high hatchability rate of incubated eggs to maintain economic viability and meet market demand (Om Shirse, 2026). Avian embryonic development is a complex, delicate physiological process governed by strict environmental parameters, of which temperature is the most critical variable (Trust, 2026).

According to the Food and Agriculture Organization (FAO, 2024), the optimal temperature range for the incubation of chicken eggs (Gallus gallus domesticus) is strictly defined between 37.0°C and 39.0°C (with a biological midpoint of 37.8°C). Embryonic tolerance to temperature fluctuations is extremely narrow compared to other livestock domains (Mohammed & Mohammed, 2025).

Sustained exposure to temperatures below 35.0°C (hypothermia) slows cellular division and developmental kinetics, leading to congenital deformities, structural abnormalities, or complete embryo death (Abraham & Stephen, 2025). Conversely, temperatures exceeding 39.5°C (hyperthermia) accelerate metabolic rates to lethal levels, causing irreversible proteins to denature and killing the embryo within 20 to 30 minutes (Abdul Hakiman Abdullah & Rohaiza Hamdan, 2023).

Figure 1.1: Biological temperature thresholds and viability zones for chicken egg incubation. Optimal development occurs strictly within the 37–39°C range.

Historically, temperature regulation in agricultural storage facilities has relied on manual inspections and analog gauges, which are prone to human error and lack real-time visibility (Okello et al., 2025). This operational vulnerability highlights the need for continuous automated monitoring systems to safeguard embryonic survival.

#### 1.1.2 The Paradigm Shift to Agriculture 4.0 and Smart Farming Systems

The integration of digital technologies into agriculture has initiated "Agriculture 4.0." This paradigm shift leverages Cyber-Physical Systems (CPS), the Internet of Things (IoT), cloud computing, and big data analytics into daily farm operations (Gatkal et al., 2025). In smart farming, physical environments are continuously monitored by networks of connected sensors, creating a feedback loop where environmental variables inform digital models (Protopappas, Bechtsis, & Tsotsolas, 2025).

As noted by Mohammed and Mohammed (2025), Agriculture 4.0 technologies allow farmers to shift from scheduled manual inspections to data-driven, precision management. By deploying IoT nodes across agricultural facilities, operators can detect micro-climate deviations immediately, optimizing environmental stability and reducing post-harvest losses.

#### 1.1.3 The Role of Internet of Things (IoT) and Mobile Applications in Environmental Automation

The Internet of Things (IoT) provides the physical infrastructure for modern environmental monitoring. Microcontrollers, such as the ESP32, act as edge processing units that interface directly with digital sensors (e.g., DS18B20 or DHT22) to capture local micro-climate data (Trust, 2026). These edge nodes convert physical parameters into digital payloads and transmit them using lightweight protocols like MQTT (Message Queuing Telemetry Transport) over local wireless networks (Abdullah & Hamdan, 2023).

Mobile applications serve as the primary human-machine interface (HMI) in these systems. By using persistent communication channels like WebSockets, mobile apps can receive telemetry streams and update visual charts in real time (Nalendra & Waspada, 2025). This allows farm operators to monitor remote storage conditions from anywhere, receiving instant push notifications if parameters drift outside safe limits (Dr. Brindha S et al., 2025).

### 1.2 Problem Statement

#### 1.2.1 Inefficiencies and Late Responses of Manual Monitoring and Standalone Gauges

Traditional poultry hatcheries rely on standalone thermometers or manual inspection schedules (Okubanjo, 2025). This approach creates large gaps in monitoring, particularly during night shifts or weekends (Om Shirse, 2026). If a heating element or ventilation fan fails, hours can pass before the issue is detected, exposing sensitive embryos to extreme temperatures and leading to high mortality rates (Okello et al., 2025).

Furthermore, manual inspection requires physically opening the incubator doors, which disrupts the internal micro-climate (humidity and temperature equilibrium) and causes further developmental stress to the embryos (Okello et al., 2025).

#### 1.2.2 Micro-climate Deviations and Resulting Economic/Post-Harvest Losses

For commercial poultry operations, temperature deviations in incubators represent a significant financial risk. A single equipment failure that goes unnoticed for even short intervals can destroy an entire batch of eggs, causing severe financial losses, disrupting poultry supply chains, and threatening the operational viability of the farm (Okubanjo, 2025; Pacheco da Costa et al., 2023).

To mitigate these losses, farms require a monitoring system with sub-2-second latency to notify off-site operators the moment a temperature threshold is breached.

#### 1.2.3 Vulnerabilities, Lack of Access Control, and Lack of Remote Capabilities in Existing Farming Apps

While various smart farming applications exist, they often suffer from significant architectural weaknesses. Many current solutions use unencrypted communication protocols (such as standard HTTP or plain MQTT) and lack robust authentication, leaving them vulnerable to data interception and spoofing attacks (Lengkong et al., 2025).

Furthermore, these applications often lack role-based access control (RBAC), allowing unauthorized users to modify temperature thresholds or disable critical alerting systems (Kale et al., 2024).

#### 1.2.4 The Security Gap: Omission of Firmware and Mobile Security Update Notifications

A major vulnerability in agricultural IoT systems is the lack of ongoing software maintenance. Microcontroller firmware and mobile applications are rarely updated once deployed, leaving them exposed to security exploits (Dr. Brindha S et al., 2025). Most agricultural monitoring systems lack a mechanism to verify software versions or notify operators when a security patch is available, leaving devices vulnerable to unauthorized access and network hijacks.

If an attacker gains control of the edge node, they can spoof telemetry data (e.g. reporting an optimal 37.8°C while the actual temperature is dropping to lethal levels), masking incubator failures and leading to total batch losses (Lengkong et al., 2025).

### 1.3 Research Aim and Objectives

#### 1.3.1 Research Aim

The primary aim of this research is to design, develop, and evaluate a secure, real-time IoT-based mobile application framework (named Egg Guardian) for monitoring poultry egg incubator temperature and delivering automated firmware and application security update notifications.

#### 1.3.2 Specific Research Objectives

To comply with standard undergraduate research limits (Research Proposal Outline, 2023), the objectives are consolidated into exactly four key areas:

- Hardware Edge Node Design: Build an ESP32-based hardware edge node that interfaces with DS18B20/DHT22 sensors and features an offline ring buffer to prevent telemetry data loss during connectivity drops.

- Unified Backend and HMI Development: Design and implement a centralized cloud backend using FastAPI to process telemetry, coupled with a cross-platform Flutter mobile application to display real-time readings and receive push notifications.

- Security Subsystem Implementation: Integrate cryptographic access controls (JWT), secure transport layers (MQTTS/TLS), and an automated edge firmware lifecycle tracking system to notify users of security updates.

- Experimental Evaluation and Validation: Benchmark and evaluate the system's end-to-end latency, database ingestion reliability, local caching capability, and the propagation speed of firmware security alerts.

#### 1.3.3 Alignment of Project Aim and Objectives with Identified Gaps

This project directly aligns its specific objectives with the identified structural gaps, as shown below: Obj 2 (Unified Backend & HMI) addresses manual check latency. Obj 1 (Offline Ring Buffer) handles data loss. Obj 3 (Security & Update Systems) mitigates spoofing and unencrypted traffic. Obj 4 (Experimental Evaluation) resolves outdated firmware security gaps.

### 1.4 Significance of the Study

#### 1.4.1 Optimization of Post-Harvest Durability and Quality for Farm Operations

By providing sub-second telemetry transmission and immediate background alerting, this system enables poultry farm operators to respond to heating or ventilation failures before embryonic damage occurs (Okubanjo, 2025). This level of environmental control maximizes hatch rates, protects animal welfare, and improves the hatchery's overall productivity (Om Shirse, 2026).

#### 1.4.2 Cost-Effective Scalability for Small-to-Mid Scale Poultry Farms

Unlike expensive proprietary systems, the proposed solution utilizes open-source frameworks (FastAPI, Flutter) and low-cost, off-the-shelf microcontrollers (ESP32). This approach allows small and mid-scale poultry hatcheries to deploy a professional monitoring solution without high licensing fees or complex installations (Trust, 2026; Abdullah & Hamdan, 2023).

#### 1.4.3 Introduction of Security and Device Integrity Awareness in Agricultural Computing

This research addresses a critical gap in smart farming by demonstrating how security updates and device lifecycle management can be integrated directly into agricultural software (Lengkong et al., 2025). This establishes a template for building secure IoT networks in the agricultural domain (Dr. Brindha S et al., 2025).

### 1.5 Scope and Limitations of the Research

#### 1.5.1 Technical Scope

The technical scope of the project encompasses a hardware layer (ESP32 microcontroller with DS18B20 or DHT22 sensor) (Trust, 2026; Shirse, 2026), transport layer (MQTT over secure MQTTS) (Abdullah & Hamdan, 2023), cloud layer (FastAPI backend with PostgreSQL), and HMI layer (Flutter application and companion web dashboard) (Nalendra & Waspada, 2025).

Figure 1.2: General systems architecture mapping the data flow from the incubator sensor node to the FastAPI cloud backend and the mobile/web interfaces.

#### 1.5.2 Operational Limits

The system is designed for stationary egg storage and incubator environments and requires a local Wi-Fi connection at the hatchery site (Okello et al., 2025). Mobile monitoring and push alerts require cellular data or internet connectivity for the operator's smartphone. The current prototype does not monitor eggs in transit during shipping or logistics (Pacheco da Costa et al., 2023).

### 1.6 Operational Definition of Technical Terms

- Agriculture 4.0: The integration of digital technologies, such as IoT and cloud computing, to automate and optimize agricultural production (Gatkal et al., 2025).

- Asynchronous I/O: A programming model that allows a server to handle multiple operations concurrently without blocking execution, critical for high-frequency sensor streams.

- Bcrypt: A secure, salt-based password hashing algorithm used to store user credentials.

- Cyber-Physical System (CPS): A system where physical mechanisms are controlled and monitored by computer-based algorithms (Protopappas et al., 2025).

- JSON Web Token (JWT): An open standard (RFC 7519) that defines a compact, secure way to transmit information between parties as a JSON object, used for user authentication.

- MQTT (Message Queuing Telemetry Transport): A lightweight publish-subscribe messaging protocol designed for resource-constrained devices and low-bandwidth networks (Abdullah & Hamdan, 2023).

- Over-The-Air (OTA): The wireless delivery of software updates or firmware patches to remote devices.

- Telemetry: The automatic measurement and wireless transmission of environmental data from remote sources.

- WebSocket: A protocol that provides full-duplex, real-time communication channels over a single TCP connection, enabling instant UI updates (Nalendra & Waspada, 2025).

## CHAPTER 2: LITERATURE REVIEW

### 2.1 Introduction

#### 2.1.1 Purpose of the Chapter

The primary purpose of this chapter is to conduct a systematic and critical review of the existing body of literature surrounding poultry egg incubation systems, micro-climate storage monitoring, Internet of Things (IoT) communication protocols, mobile human-machine interfaces (HMIs), and edge-to-cloud security frameworks. By exploring current research, this review establishes the academic and technical foundation for the proposed Egg Guardian system.

#### 2.1.2 Scope of the Literature Reviewed

The scope of this literature review encompasses 29 peer-reviewed journal articles, systematic reviews, conference proceedings, and technical proposals published between 2015 and 2026. The materials are categorized into four core domains:

- Biological Constraints & Poultry Environmental Systems: Focuses on incubation temperature and relative humidity tolerances.

- Agricultural IoT Topologies: Reviews microcontrollers (ESP32, ESP8266), data transmission protocols (MQTT, HTTP), and network caching mechanisms.

- Mobile HMI Paradigms: Examines visual dashboarding, real-time push engines, and operating system background execution limits.

- IoT Security Architectures: Explores transport encryption, API token access controls, and microcontroller firmware lifecycle update vulnerabilities.

#### 2.1.3 Overview of the Chapter Structure

This chapter is organized into eight logical sections. Section 2.1 provides this introductory scope. Section 2.2 conducts a Conceptual Review of smart farming technologies and embryonic constraints. Section 2.3 presents a Review of Existing Systems, highlighting their operational limitations. Section 2.4 details the Theoretical Framework (Queuing Theory, TLS cryptographic models, and Role-Based Access Control). Section 2.5 outlines the Conceptual Framework using a variables relationship diagram. Section 2.6 presents the Empirical Review of 14 core studies, summarized in a thematic comparative matrix. Section 2.7 identifies the Research Gaps, and Section 2.8 provides a summary of the reviewed literature.

### 2.2 Conceptual Review

#### 2.2.1 Definition of the Problem

Commercial poultry hatcheries depend on achieving a high hatchability rate of incubated eggs to maintain economic viability (Om Shirse, 2026). Avian embryonic development is governed by strict environmental parameters, of which temperature is the most critical variable (Trust, 2026). The optimal temperature range for the incubation of chicken eggs (Gallus gallus domesticus) is strictly defined between 37.0°C and 39.0°C (with a biological midpoint of 37.8°C) (FAO, 2024).

Embryonic tolerance to temperature fluctuations is extremely narrow. Sustained exposure to temperatures below 35.0°C (hypothermia) slows cellular division and developmental kinetics, leading to congenital deformities or embryo death (Abraham Atosona & Stephen, 2025). Conversely, temperatures exceeding 39.5°C (hyperthermia) accelerate metabolic rates to lethal levels, causing irreversible proteins to denature and killing the embryo within 20 to 30 minutes (Abdul Hakiman Abdullah & Rohaiza Hamdan, 2023).

Historically, temperature regulation has relied on manual inspections and analog gauges, which are prone to human error and lack real-time visibility (Okello et al., 2025). This operational vulnerability highlights the need for continuous automated monitoring systems to safeguard embryonic survival.

#### 2.2.2 Historical Background of Smart Farming (Agriculture 4.0)

The integration of digital technology into the agricultural sector has transformed traditional practices, giving rise to "Agriculture 4.0." Within this paradigm, the Agricultural Cyber-Physical Domain plays a critical role by linking physical farming operations with cloud-based computation (Gatkal et al., 2025). Cyber-Physical Systems (CPS) in agriculture consist of sensor networks, communication interfaces, and cloud engines that monitor physical states and coordinate responses (Protopappas, Bechtsis, & Tsotsolas, 2025).

As noted by Mohammed and Mohammed (2025), Agriculture 4.0 technologies allow farmers to shift from scheduled manual inspections to data-driven, precision management. By deploying IoT nodes across agricultural facilities, operators can detect micro-climate deviations immediately, optimizing environmental stability and reducing post-harvest losses.

Figure 2.1: The Cyber-Physical feedback loop in smart agriculture, showing the cycle of physical sensing, cloud analysis, and automated alerting/control.

#### 2.2.3 Key Technologies

- ESP32 Microcontroller: A low-cost, low-power system-on-a-chip microcontroller with integrated Wi-Fi and dual-mode Bluetooth (Trust, 2026). It serves as the edge node processing unit.

- MQTT Protocol: Message Queuing Telemetry Transport is a lightweight publish-subscribe messaging protocol designed for resource-constrained devices and low-bandwidth networks (Abdullah & Hamdan, 2023).

- WebSockets vs. HTTP Polling: HTTP polling relies on periodic HTTP requests to check for new data, creating high network overhead. In contrast, WebSockets provide a persistent, full-duplex communication channel over a single TCP connection, enabling real-time UI updates (Nalendra & Waspada, 2025).

- FastAPI Framework: An asynchronous, high-performance web framework for building APIs with Python, utilizing standard Python type hints to process sensor streams without blocking.

- JWT Access Control: JSON Web Tokens provide a compact and self-contained method for securely transmitting information between parties as a JSON object, securing API endpoints (Lengkong et al., 2025).

- Firebase Cloud Messaging (FCM): A cross-platform messaging solution that lets you reliably send background push notifications to user devices (Dr. Brindha S et al., 2025).

Figure 2.3: Data flow comparison of HTTP polling (introducing significant request overhead and delays) vs. persistent WebSockets (enabling instant, low-overhead updates).

#### 2.2.4 Components of the Proposed System

The proposed Egg Guardian system consists of four primary components:

- Hardware Edge Node: An ESP32 microcontroller interfaced with a DHT22/DS18B20 sensor. It incorporates a local RAM/SQLite-based ring buffer to store up to 20 telemetry records during Wi-Fi drops.

- Transport Layer: Encrypted MQTTS (port 8883) using TLS wraps for telemetry transmission.

- Centralized Cloud Backend: A FastAPI server that ingests telemetry, manages PostgreSQL storage, checks edge firmware versions, and evaluates threshold rules.

- Mobile HMI Application: A Flutter application that visualizes real-time sensor charts via WebSockets and receives background alerts via FCM.

Figure 2.4: Secure MQTT Publish-Subscribe architecture showing the implementation of TLS encryption over port 8883 to prevent man-in-the-middle attacks.

#### 2.2.5 Advantages and Limitations of Existing Technologies

Traditional localized setups (using Arduino Uno or basic Raspberry Pi web servers) are simple to build but suffer from significant limitations (Kale et al., 2024). They lack cloud scalability, remote background notification systems, and transport encryption. Meanwhile, commercial platforms (like Blynk or ThingSpeak) offer rapid prototyping but lock users into proprietary clouds, charge high licensing fees, lack local edge buffering during network outages, and omit device firmware security update checks (Okello et al., 2025).

#### 2.2.6 Current Trends

Current trends in agricultural computing focus on Edge-Cloud collaboration, where edge nodes perform local filtering and caching while cloud backends handle heavier processing and notifications (Gatkal et al., 2025). Additionally, there is an increasing emphasis on User-Centric Design (UCD)—incorporating sleek interfaces, dark mode styling, and real-time WebSocket charts—to improve human operator adoption and trust in automation.

### 2.3 Review of Existing Systems

#### 2.3.1 System A: GSM-SMS Notification Incubators

This class of systems utilizes microcontrollers (e.g., Arduino Uno) connected to a GSM shield (e.g., SIM800L) to send SMS notifications during threshold breaches (Abraham Atosona & Stephen, 2025).

- Features: SMS-based threshold alerting, basic local relay temperature control.

- Strengths: Operates in regions without internet connectivity by utilizing cellular networks.

- Weaknesses: High SMS operational costs, SMS delivery delays, no real-time telemetry charts.

#### 2.3.2 System B: Localized Raspberry Pi Gateway Servers

These setups use localized computers (like a Raspberry Pi) hosted at the farm to act as the database and local web server (Kale et al., 2024).

- Features: Local SQLite logging, local dashboard rendering.

- Strengths: Operates completely offline without internet dependency.

- Weaknesses: Limited remote accessibility, high local hardware costs, vulnerability to local power outages.

#### 2.3.3 System C: Unencrypted Cloud Dashboards (e.g., Blynk Platform)

These systems integrate microcontrollers directly with third-party cloud engines to push telemetry to mobile dashboards (Okello et al., 2025).

- Features: Cloud database serialization, mobile dashboard widget templates.

- Strengths: Rapid deployment, minimal coding required for mobile layouts.

- Weaknesses: Data loss during Wi-Fi outages (no edge buffering), unencrypted data transit, lack of access control, no firmware version verification.

#### 2.3.4 Comparative Matrix of Existing Systems and Proposed Egg Guardian

| System Type | Features | Strengths | Limitations | Comparison with Egg Guardian |
| --- | --- | --- | --- | --- |
| GSM-SMS Systems (Abraham & Stephen, 2025) | SMS Alerts, local control | No internet needed | High SMS costs, no UI, high latency | Egg Guardian provides low-cost WebSocket streams and FCM pushes. |
| Local Gateway Servers (Kale et al., 2024) | Local database, local UI | Completely offline | High cost, complex wiring, no cloud scaling | Egg Guardian utilizes cloud-native FastAPI backend with PostgreSQL. |
| Blynk Cloud Systems (Okello et al., 2025) | Cloud logs, template UI | Rapid setup | No offline caching, proprietary, insecure | Egg Guardian integrates a local SQLite/RAM buffer and secure MQTTS. |
| Egg Guardian (Proposed System) | Real-time charts, FCM alerts, JWT, MQTTS, Edge Buffering, Firmware lifecycle checks | Zero telemetry loss on network drop, JWT access control, MQTTS/TLS, low-cost | Requires local Wi-Fi and mobile data links | Focus of validation. |

### 2.4 Theoretical Framework

#### 2.4.1 Queuing Theory and Edge Caching Models (M/M/1 Queue Model)

This study utilizes Queuing Theory to model the transmission of sensor telemetry over unstable wireless networks. Telemetry generation at the edge node can be modeled as a Poisson process with arrival rate λ, while transmission to the cloud is a service process with rate μ. In a stable network, λ < μ, and the queue remains empty. However, during network drops, μ drops to zero, representing a service interruption.

- Assumption: Telemetry packets are generated at fixed sampling intervals (t = 5s) and stored in a FIFO (First-In, First-Out) queue.

- Equation: The probability of buffer overflow in a finite queue of capacity K during an outage of duration D is modeled by:

Poverflow = P(N(D) ≥ K)

where N(D) is the number of telemetry points generated during the outage.

- Application: This theory supports the implementation of the offline ring buffer, proving that a buffer capacity of K=20 is mathematically sufficient to prevent telemetry loss during Wi-Fi drops of up to 100 seconds.

#### 2.4.2 Transport Layer Security (TLS 1.3) Cryptographic Model

The TLS 1.3 cryptographic model provides the theoretical foundation for secure edge-to-cloud communications. It utilizes asymmetric cryptography for key exchange (Diffie-Hellman), symmetric cryptography for bulk data encryption (AES-256), and cryptographic hashing (SHA-256) for data integrity.

- Assumption: The edge node has the server's CA certificate pre-flashed, preventing Man-in-the-Middle (MitM) attacks.

- Application: This model protects telemetry data from eavesdropping and tampering during transit over public networks (Lengkong et al., 2025).

#### 2.4.3 Role-Based Access Control (RBAC) and Token-Based Authentication Model

The JWT access control model relies on the Token-Based Authentication theory. Upon providing valid credentials, the user receives a cryptographically signed token containing claims (subject, role, expiration).

- Assumption: The server validates the token's signature using a secret key without querying the database, reducing database overhead.

- Application: This model secures API routes, ensuring that only authenticated operators can modify incubator thresholds or access alert configurations.

### 2.5 Conceptual Framework

#### 2.5.1 Conceptual Framework Diagram

The conceptual framework maps the relationships between the independent, dependent, and moderating variables of the Egg Guardian system.

Figure 2.5: Conceptual Framework diagram, showing the relationship between independent variables (sensor precision, edge buffering, encryption, version checks), dependent variables (hatchability, zero telemetry loss, data integrity, update awareness), and moderating variables (network reliability, power stability).

#### 2.5.2 Explanation of the Diagram

The framework consists of:

- Independent Variables (System Inputs): The design features of the proposed system:

  - Continuous Temperature & Humidity Sensing: Constant environmental monitoring.

  - Offline Edge Ring Buffering: Local caching during network drops.

  - JWT Access Control & MQTTS-TLS Encryption: System security measures.

  - Automated Edge Version Verification: Firmware lifecycle management.

- Dependent Variables (System Out-of-the-Box Outcomes): The target metrics:

  - Optimized Embryonic Hatchability Rate: Preserving embryonic viability.

  - Zero Telemetry Log Loss: Complete environmental records.

  - Telemetry Data Integrity & Operator Trust: Protecting against data tampering.

  - Operator Update Awareness: Notifying users of security updates.

- Moderating Variables (Environmental Factors): Factors that influence system performance:

  - Local Power Grid Stability: Power availability at the farm.

  - Rural Wi-Fi Network Reliability: Wireless signal strength.

### 2.6 Empirical Review

#### 2.6.1 Detailed Review of Core Empirical Studies

The empirical review examines historical deployments of smart incubator and poultry management systems:

- Protopappas, Bechtsis, & Tsotsolas (2025): Evaluated IoT architectures for food cold chains. Adopted a cloud-based ingestion backend. Found that real-time monitoring reduced food waste by 22%. Limitation: Did not address edge-node security or update notifications. Relevance: Informs cloud ingestion logic.

- Pacheco da Costa et al. (2023): Reviewed 45 agricultural WSN papers. Identified that 65% of WSN deployments suffered from packet drops. Limitation: Offered no concrete edge buffering algorithm. Relevance: Supports the need for the offline ring buffer.

- Nalendra & Waspada (2025): Implemented WebSockets in a broiler farm app. Achieved latency of under 500ms for UI updates. Limitation: No background execution; alerts silenced when app closed. Relevance: Guides WebSocket integration.

- Gatkal et al. (2025): Conducted a systematic review of WSN topologies in smart farming. Limitation: Lacked implementation details. Relevance: Connects WSN design with smart farming.

- TTI Cold Chain Study (2024): Explored time-temperature data exchanges in food supply chains. Limitation: Evaluated logistics rather than stationary environments. Relevance: Demonstrates the importance of continuous logging.

- Terence, Immaculate, Raj, & Nadarajan (2024): Systematic review of livestock monitoring CPS. Limitation: Conceptual paper without source code validation. Relevance: Supports the cyber-physical system design.

- Okello, Ahimbisibwe, & Tukamuhebwa (2025): Designed an ESP32 Blynk incubator. Limitation: Gaps in data logs during Wi-Fi drops, no security measures. Relevance: Establishes baseline ESP32/Blynk incubator performance.

- Abraham Atosona & Stephen (2025): Simulated a GSM smart incubator. Limitation: High SMS latency, lack of visual interface. Relevance: Supports the move to internet-based protocols.

- Abdullah & Hamdan (2023): Built an MQTT incubator monitor. Limitation: No background alerts, lacked access control. Relevance: Informs MQTT topic structures.

- Noor Abdullah Mohammed & Ziad Saeed Mohammed (2025): Reviewed automated poultry management systems. Limitation: Lacked security analyses. Relevance: Details target environmental thresholds.

- Okubanjo (2025): Proposed a sustainable poultry WSN. Limitation: Conceptual model, did not implement mobile dashboard. Relevance: Focuses on open-source, low-cost microcontrollers.

- Trust (2026): Implemented an ESP32 incubator. Limitation: No transport encryption, did not address firmware updates. Relevance: Guides DS18B20 1-Wire sensor wiring.

- Dr. Brindha S et al. (2025): Used AIoT for poultry monitoring. Limitation: Did not address firmware security updates. Relevance: Supports the integration of push notifications.

- K. Ramanan (2025): Automated temperature regulation in poultry farms. Limitation: High installation cost, restricted adoption. Relevance: Highlights the need for low-cost, open-source solutions.

- Sahoo & Pattnaik (2019): Built an IoT poultry farm system focusing on relay adjustments. Limitation: Lacked cloud logging or remote HMI. Relevance: Highlights the importance of localized backup triggers.

- TTI Cold Chain Study (2024): Discussed continuous temperature logging exchanges. Limitation: Evaluated food supply chain logistics rather than egg incubation. Relevance: Validates temperature logging compliance metrics.

- Nsengiyumva Wilberforce (2024): Proposed a smart agricultural framework using microcontrollers in low-resource environments. Limitation: No background execution or security model. Relevance: Confirms the viability of ESP32 platforms.

- Trust (2026): Fabricated an ESP32-based environmental chamber. Limitation: Lacked MQTTS wrapping and updates verification. Relevance: Direct predictor of hatching success rates.

- Broiler Monitoring Journal (2024): Tested temperature/humidity controls in large-scale chicken coops. Limitation: High complexity, no mobile integration. Relevance: Informs relative humidity target boundaries.

- Kumar & Singh (2018): Reviewed local web servers for agricultural parameters monitoring. Limitation: High hardware crash rate. Relevance: Justifies the transition to FastAPI cloud hosting.

- Verma & Ranga (2018): Analyzed security gaps in WSN nodes. Limitation: Purely theoretical paper. Relevance: Informs access control architectures.

- Wortmann & Flüchter (2015): Explored IoT lifecycle systems and remote updates. Limitation: Enterprise-focused, expensive. Relevance: Shapes firmware version notification model.

- Soeb et al. (2021): Fabricated a low-cost incubator. Limitation: Lacked wireless connectivity or remote UI. Relevance: Confirms developmental temperature baselines.

- Malaysia IoT Study (2020): Demonstrated a simple coops monitor using Blynk. Limitation: Network drops resulted in data gaps. Relevance: Informs the design of offline buffer queues.

#### 2.6.2 Thematic Summary Matrix (25 Core Papers Synthesis)

The following comparative matrix synthesizes the systems architectures, alerting, buffering, security postures, and identified gaps across 25 core papers reviewed in this study:

| Source Author & Year | System Core Stack | Real-time Alerting | Offline Buffer | Security Model | Gaps Identified |
| --- | --- | --- | --- | --- | --- |
| Abraham & Stephen (2025) | Arduino + GSM SMS | SMS Alerts (High Latency) | No | None | High operating SMS costs, no UI |
| Okello et al. (2025) | ESP32 + Blynk App | Blynk Push | No | Proprietary Blynk Token | No offline storage, data lost on Wi-Fi drop |
| Lengkong et al. (2025) | ESP8266 + HTTP API | Email (polling) | No | Static API Key | Vulnerable to key theft, high latency |
| Kale et al. (2024) | Raspberry Pi + Arduino | Web Server | No | Basic HTTP Auth | No cloud scalability, complex local wiring |
| Abdullah & Hamdan (2023) | ESP32 + MQTT | Dashboard View | No | None | No notification trigger, lacks secure auth |
| Shirse (2026) | ESP8266 + DHT22 | Local LED/Buzzer | No | None | No mobile app interface, lacks remote notifications |
| Gatkal et al. (2025) | Microcontrollers + WSN | Local Display | No | None | General agricultural review, no mobile app focus |
| Terence et al. (2024) | WSN + Cloud CPS | Web Dashboard | No | Basic SSL | Review paper, does not implement physical solution |
| Nalendra & Waspada (2025) | ESP32 + WebSockets | In-app alerts | No | Static token | No offline data storage, lacks background alert |
| Brindha et al. (2025) | AIoT + Cloud | Mobile Push | No | Basic HTTPS | Does not cover firmware version security updates |
| Sahoo & Pattnaik (2019) | Arduino + WSN | Local relay control | No | None | No cloud logging, lacks remote mobile HMI |
| Wolfert et al. (2017) | Big Data Framework | Cloud Dashboard | No | Standard SSL | Conceptual smart farming review, no edge implementation |
| Kamilaris et al. (2017) | Big Data Stack | Analysis Reports | No | None | Focuses on data mining rather than real-time edge control |
| Ray et al. (2017) | IoT Gateways | Basic Alerting | No | None | Early literature review, lacks secure transport details |
| TTI Study (2024) | Time-Temperature Indicators | None | Passive indicator | Physical tag | No active telemetry, no real-time mobile alerts |
| Wilberforce (2024) | Arduino + Wi-Fi | Local screen | No | None | Lacks secure cloud database logging and mobile app alerts |
| Trust (2026) | ESP32 + DS18B20 | Local Buzzer | No | WPA2 | Omites transport layer encryption (MQTTS) and remote update notifications |
| K. Ramanan (2025) | PLC + Industrial Gateway | SCADA alarm | Yes | Password | High deployment cost, proprietary architecture, lacks consumer mobile HMI |
| Broiler Journal (2024) | WSN + Gateway | Local buzzer | No | None | Focuses on physical measurements rather than secure remote app access |
| Kumar & Singh (2018) | Arduino + Web | None | No | HTTP Auth | High risk of local server failures, no mobile push notification layer |
| Verma & Ranga (2018) | Conceptual Security | None | None | Crypto review | Review paper; does not implement a physical incubator sensing prototype |
| Wortmann & (2015) | Industrial IoT | Dashboard | Yes | Certificate | Heavy enterprise focus, unsuitable for low-cost smallholder farming systems |
| Soeb et al. (2021) | Microprocessor + Relays | LCD warning | No | None | Lacks wireless connectivity, cloud storage, or remote monitoring interfaces |
| Malaysia Study (2020) | NodeMCU + Blynk | Blynk Push | No | Blynk Token | No offline data resilience, lacks transport-level TLS encryption |
| This Study (Egg Guardian) | ESP32 + FastAPI + Flutter | FCM Push + WebSocket + Gmail API | Yes (20 records) | JWT, TLS, Security Update Alerts | None (Focus of validation) |

### 2.7 Research Gap

#### 2.7.1 The Isolation of Smart Environmental Architecture from Mobile App Security

In current agricultural IoT literature, system design and security are treated as separate concerns. Projects focus on temperature precision while leaving data channels unencrypted and access unauthenticated.

#### 2.7.2 Absence of Low-Cost, Secure Storage Platforms for Developing Agricultural Economies

There is a lack of end-to-end, secure, open-source systems specifically designed to run on resource-constrained networks. The development of a secure, lightweight, and offline-resilient platform using standard frameworks like FastAPI and Flutter represents a clear contribution to the field.

Figure 2.2: Security vulnerability map for agricultural IoT networks, highlighting unauthenticated nodes, plaintext transit, outdated firmware, and weak access controls.

### 2.8 Summary of Literature

This chapter reviewed the literature surrounding agricultural cyber-physical domains (Gatkal et al., 2025), WSN buffering models (Pacheco da Costa et al., 2023), real-time WebSockets (Nalendra & Waspada, 2025), and IoT security vulnerabilities (Lengkong et al., 2025). The theoretical framework established Queuing Theory to model offline caching and TLS/JWT to secure communication channels. The conceptual framework diagrammed independent, dependent, and moderating variables. The empirical review highlighted gaps in data logging continuity and security lifecycle maintenance in existing architectures. These findings inform the development of the Egg Guardian system, ensuring a secure, reliable, and low-cost solution for poultry egg storage monitoring.

## CHAPTER 3: RESEARCH METHODOLOGY & SYSTEM DESIGN

### 3.1 Introduction

This chapter explains the methodology used in designing, developing, implementing, and testing the proposed mobile application for poultry egg temperature monitoring and security update notifications. The Egg Guardian system integrates physical microcontrollers, cloud APIs, and a mobile frontend to establish a secure, real-time cyber-physical loop (Protopappas, Bechtsis, & Tsotsolas, 2025). The following sections detail the research design, system development life cycle phases, functional and non-functional requirements, architectural topologies, UML design diagrams, hardware wiring schematics, and testing criteria.

### 3.2 Research Design

This study adopts a hybrid research design combining Design and Development Research (DDR) (Richey & Klein, 2014) with Experimental Research (Campbell & Stanley, 2015).

- Design and Development Research: Focusing on the systematic study of designing, developing, and evaluating software systems, DDR provides a structured framework for translating requirements into functional cyber-physical components (Hevner, March, Park, & Ram, 2004). This design is highly appropriate for developing a mobile application since it guides the engineering lifecycle from architectural modeling to active HMI layouts.

- Experimental Research: Involves testing the physical subsystem under varying environmental thermal loads and network connectivity states. This allows the study to empirically measure sensor accuracy (MAE) and packet recovery rates during simulated Wi-Fi drops (Pacheco da Costa et al., 2023).

### 3.3 System Development Methodology

This study utilizes the Iterative Development Model (Larman & Basili, 2003) to build, refine, and validate the software components. The development lifecycle is structured into six phases:

- Requirement Analysis: Translating biological temperature constraints and security lifecycle needs into hardware and software specifications (Gatkal et al., 2025).

- System Design: Modeling database structures, message-passing sequences, use case flows, and hardware schematics.

- Development: Writing the C++ firmware for the ESP32, constructing the FastAPI server using Python, and building the Flutter mobile UI using Dart.

- Testing: Executing unit tests on API endpoints, testing MQTT broker transport latency, and verifying FCM notification delivery.

- Deployment: Uploading the backend API to the cloud, distributing the Flutter mobile app build, and flashing firmware onto edge microcontrollers.

- Maintenance: Monitoring historical logs, adjusting threshold hysteresis values, and pushing firmware security update notification strings.

### 3.4 Requirement Analysis

#### 3.4.1 Functional Requirements

The system must execute the following functions to meet user needs:

- User Registration and Login: Provide secure account creation and session authorization using JSON Web Tokens (JWT) (Lengkong et al., 2025).

- Display Real-Time Egg Storage Temperature: Visualize current temperature readings captured by the DS18B20 digital sensor (Trust, 2026).

- Display Security Status: Render device update alerts and firmware version mismatch warning banners in the mobile UI.

- Push Notifications for Abnormal Temperature: Trigger system-level FCM alarms when temperatures cross the safe boundaries of 37.0°C to 39.0°C (FAO, 2024).

- Security Update Notifications: Notify the operator when a firmware version mismatch is detected in the backend broker (Dr. Brindha S et al., 2025).

- Temperature History: Render scrollable charts of historical temperature telemetry logged over time.

- User Profile Management: Allow operators to modify threshold limits, user passwords, and notification preferences.

#### 3.4.2 Non-Functional Requirements

- Fast Response Time: End-to-end telemetry propagation and FCM alert notification delivery must take less than 2.0 seconds (Nalendra & Waspada, 2025).

- Secure Authentication: Passwords must be hashed using Bcrypt, and API route communication must require JWT validation (Verma & Ranga, 2018).

- High Availability: The FastAPI server must handle asynchronous non-blocking connection requests from multiple edge nodes and mobile clients concurrently.

- User-Friendly Interface: The Flutter application must incorporate modern UX practices, intuitive color scales (representing thermal zones), and clear alert banners.

- Scalability: The backend database must support writing and reading historical telemetry logs across multiple facilities.

- Reliability: The system must maintain data completeness during Wi-Fi drops by caching logs on the edge node buffer and flushing them on reconnect (Pacheco da Costa et al., 2023).

### 3.5 System Architecture

#### 3.5.1 System Architecture Topology

The physical and virtual components are connected in a multi-tier cyber-physical topology.

Temperature Source (DS18B20 Probe)

│

▼

ESP32 Edge Node (Buffer Queue)

│

▼ (MQTT TLS Port 8883)

Backend/API (FastAPI Server)

│

├────────────────────────┐

▼                        ▼

Database (PostgreSQL)     Push Notification Service (FCM)

│

▼

Mobile Application (Flutter UI)

Data flows through this architecture in a continuous sequence:

- The DS18B20 digital temperature probe captures thermal readings and transmits them to the ESP32 microcontroller (Trust, 2026).

- The ESP32 publishes the telemetry data over an encrypted MQTT TLS channel (port 8883) to the FastAPI Backend/API (Abdullah & Hamdan, 2023).

- The FastAPI server processes the payload, checks edge firmware versions, writes the record to the PostgreSQL Database, and broadcasts the data to active clients via WebSockets.

- If the temperature violates configured limits, the FastAPI backend sends an alert payload to the Firebase Cloud Messaging (FCM) Service, which pushes a system-level alarm to the Flutter Mobile Application (Dr. Brindha S et al., 2025).

Figure 3.1: Layered system architecture of the Egg Guardian system, illustrating the Physical/Sensing, Transport, Cloud Service, and HMI/Mobile layers.

### 3.6 System Design

#### 3.6.1 Use Case Diagram

Figure 3.5: Use Case Diagram for the Egg Guardian mobile app.

#### 3.6.2 Activity Diagram

Figure 3.6: Activity Diagram showing system processing logic for network drops and alert states.

#### 3.6.3 Sequence Diagram

Figure 3.2: Asynchronous UML Data Flow Sequence Diagram, illustrating message exchanges between the ESP32 node, MQTT broker, FastAPI backend, PostgreSQL database, and the Flutter mobile application via WebSockets and FCM.

#### 3.6.4 Entity Relationship Diagram (ERD)

Figure 3.3: Entity-Relationship database schema diagram showing the tables, primary/foreign keys, and relational cardialities.

#### 3.6.5 Hardware Wiring Schematic

Figure 3.4: Hardware wiring schematic showing the ESP32 NodeMCU, DHT22 and DS18B20 sensors, pull-up resistors, relay board, and AC heating/cooling actuators.

### 3.7 Development Tools and Technologies

#### 3.7.1 Mobile Development

Flutter (Dart): An open-source UI software development kit created by Google, used to compile the cross-platform Android and iOS application from a single codebase. It provides high-performance WebSocket listeners and charts widgets.

#### 3.7.2 Backend

FastAPI (Python): A modern, fast (high-performance) web framework for building APIs with Python 3.11. Pydantic handles validation, and Starlette handles ASGI routing, enabling asynchronous non-blocking connection processing.

#### 3.7.3 Database

PostgreSQL: A powerful, open-source object-relational database system, utilized as the central repository to log historical temperature records, security alerts, and user profiles.

#### 3.7.4 Programming Languages

- Dart: Used for mobile UI widgets layouts and WebSocket client routines.

- Python: Used for FastAPI endpoint logic, security checks, and database queries.

- C++ (Arduino IDE): Used to write the ESP32 firmware, managing the local sensor query loop, offline buffering, and MQTT connections.

#### 3.7.5 Development Tools

- Android Studio: The official integrated development environment (IDE) for Google's Android operating system, used to emulate mobile runs.

- Visual Studio Code: A lightweight code editor used for writing Python backend API routines and Dart scripts.

- GitHub: A Web-based Git repository hosting service used for source code version control and collaborative backups.

### 3.8 Database Design

#### 3.8.1 Database Schema and Tables

The database consists of four tables designed to store telemetry logs, credentials, and alerts:

#### Table 3.1: Users Table Schema

| Attribute Name | Data Type | Constraints | Description |
| --- | --- | --- | --- |
| id | SERIAL | Primary Key | Unique internal user identifier |
| email | VARCHAR(255) | Unique, Not Null | User login credentials email address |
| password_hash | VARCHAR(255) | Not Null | Bcrypt hashed session password |
| role | VARCHAR(50) | Not Null | Authorization scope (Operator/Admin) |

#### Table 3.2: Temperature Records Table Schema

| Attribute Name | Data Type | Constraints | Description |
| --- | --- | --- | --- |
| id | SERIAL | Primary Key | Unique record identifier |
| sensor_id | VARCHAR(100) | Not Null | Hardware DS18B20 digital address string |
| temperature | NUMERIC(5, 2) | Not Null | Captured decimal temperature reading (°C) |
| timestamp | TIMESTAMP | Not Null | Database record ingestion time |

#### Table 3.3: Security Alerts Table Schema

| Attribute Name | Data Type | Constraints | Description |
| --- | --- | --- | --- |
| id | SERIAL | Primary Key | Unique warning alert identifier |
| alert_type | VARCHAR(100) | Not Null | Alert category string (e.g. Firmware Mismatch) |
| message | TEXT | Not Null | User-facing descriptive notification payload |
| timestamp | TIMESTAMP | Not Null | Occurrence timestamp of security event |

#### Table 3.4: Notifications Table Schema

| Attribute Name | Data Type | Constraints | Description |
| --- | --- | --- | --- |
| id | SERIAL | Primary Key | Unique identifier for notification track |
| user_id | INTEGER | Foreign Key | Reference mapping to Users PK (id) |
| title | VARCHAR(255) | Not Null | Mobile notification alert title header |
| body | TEXT | Not Null | Mobile notification alert body content |
| status | VARCHAR(50) | Not Null | Status flag (Delivered / Pending / Read) |

### 3.9 System Implementation

#### 3.9.1 Login Page

The login page serves as the entry point for the mobile application. Operators input their email and password, which the Flutter app sends to the backend via HTTPS. If verified, the app receives a JWT token and saves it to secure storage to authorize subsequent requests.

#### 3.9.2 Dashboard

The Dashboard acts as the primary cockpit for the poultry operator. It initializes WebSocket connections to the cloud backend to stream live temperature data and registers the FCM token to enable push notifications (Nalendra & Waspada, 2025).

#### 3.9.3 Temperature Monitoring Screen

This screen displays real-time temperature readings on a color-coded gauge (Green: safe, Red: abnormal). It includes a historical line chart powered by the Flutter Syncfusion charting library, showing temperature trends over a 24-hour window.

#### 3.9.4 Notification Screen

Displays a scrollable log of historical security update alerts and temperature threshold warnings, retrieving records from the backend API.

#### 3.9.5 Settings Page

Allows farm administrators to adjust temperature threshold boundaries (Tmin, Tmax) and toggle push notification preferences.

#### 3.9.6 Backend Integration

The integration connects the mobile app with the backend using WebSockets for real-time telemetry streaming and REST APIs for authentication and settings management.

### 3.10 Testing and Validation

#### 3.10.1 Testing Methods

- Unit Testing: Validating isolated functions, such as the ESP32 ring buffer enqueue logic and backend JWT signature parsing.

- Integration Testing: Verifying the communication links between the ESP32 edge node, MQTT broker, and FastAPI backend API.

- System Testing: Testing the end-to-end system from physical sensor read to mobile UI update.

- User Acceptance Testing (UAT): Under UAT, farm operators navigate the app to verify dashboard usability, historical chart readability, and alert responsiveness.

#### 3.10.2 UAT Survey Structure (System Usability Scale)

The User Acceptance Testing is evaluated using the standardized 10-item Likert-scale System Usability Scale (SUS) questionnaire (Brooke, 1996). Ten operators evaluate the application across statements such as: "I found the system very easy to use" and "I would need the support of a technical person to be able to use this app."

The final SUS score (Stotal) is computed as follows:

- For odd-numbered questions (i ∈ {1, 3, 5, 7, 9}):

Si = Xi - 1

where Xi is the user score (1 to 5).

- For even-numbered questions (i ∈ {2, 4, 6, 8, 10}):

Si = 5 - Xi

- The cumulative score is scaled to a value out of 100:

Stotal = 2.5 × ∑ Si

A threshold score of Stotal ≥ 70 is required to validate system readiness for field operations.

#### 3.10.3 Evaluation Criteria

- Accuracy of Temperature Updates: The DS18B20 sensor readings must align with a reference thermometer, maintaining an MAE ≤ 0.5°C (Trust, 2026).

- Notification Delivery: Notifications must arrive within 2.0 seconds of a threshold breach (Nalendra & Waspada, 2025).

- Response Time: WebSocket updates must refresh the dashboard charts in under 500ms.

- Usability: The app must achieve an average System Usability Scale (SUS) score above 70 during UAT trials.

- Reliability: The ESP32 local buffer must capture all telemetry points during Wi-Fi drops and flush them with 100% data recovery on reconnect (Pacheco da Costa et al., 2023).

### 3.11 Ethical Considerations

#### 3.11.1 User Privacy

No personal data beyond the user's email address is collected. This minimizes the risk of exposing sensitive user information.

#### 3.11.2 Data Security

All API route communications are secured with HTTPS and JWT authorization tokens. This ensures only authorized users can read historical logs or modify temperature configurations.

#### 3.11.3 Secure Authentication

User passwords are encrypted with Bcrypt before storage in the PostgreSQL database, protecting user credentials in the event of a database compromise.

#### 3.11.4 Data Confidentiality

Telemetry data is transmitted over encrypted channels (MQTTS/TLS port 8883) to prevent unauthorized interception.

### 3.12 Summary

This chapter explained the design and development methodology of the Egg Guardian system. It described the iterative development model, functional and non-functional requirements, system architecture, UML design diagrams, hardware wiring schematics, and testing criteria. The next chapter presents the implementation, results, and experimental evaluation.

## CHAPTER 4: SYSTEM IMPLEMENTATION, TESTING, & EVALUATION

### 4.1 Introduction

This chapter presents the physical implementation, software deployment walkthroughs, and empirical testing of the Egg Guardian system. The system integrates the ESP32 edge hardware node, FastAPI web backend, and Flutter cross-platform mobile interface to manage poultry egg climates without packet loss (Nsengiyumva Wilberforce, 2024). The following sections detail the configuration of implementation environments, walkthroughs of HMI screens, code fragments, sensor drift calibrations, real-time alert latency distribution graphs, network buffer recovery statistics, security check verifications, and UAT usability scores.

### 4.2 Development and Implementation Environment

The implementation workspace is partitioned into three environment segments:

- Edge Node Compilation: Developed using PlatformIO inside Visual Studio Code. The firmware leverages the ArduinoJson library for serialization and the DallasTemperature library for 1-Wire DS18B20 communications (Trust, 2026).

- API Backend Engine: Written in Python 3.11 with FastAPI (Starlette routing framework). The backend runs on Uvicorn (ASGI web server) and interfaces with a PostgreSQL instance using SQLAlchemy asynchronous sessions.

- HMI Handset Emulation: Developed in Dart using Flutter SDK (Target API Level 33+), running on physical Android devices.

### 4.3 Software Implementation Walkthrough

#### 4.3.1 Mobile App User Interfaces (Android Client HMI)

The mobile app gives operators real-time visibility into storage conditions (Nalendra & Waspada, 2025):

- Authentication: The login screen enforces secure access using JWT signatures (Lengkong et al., 2025).

- Dashboard & Device List: Lists all active monitoring crates, displaying connection states and current temperatures.

- Temperature Gauge details: Shows real-time thermal curves and color-coded status circles (Green: safe, Red: abnormal).

- Security Notification Alerts: Logs firmware update status flags and temperature warning histories.

Figure 4.1: Mobile Login Authentication page.

Figure 4.2: Mobile Device List Screen.

Figure 4.3: Mobile Device selection dashboard.

Figure 4.4: Real-time Temperature details monitoring gauge.

Figure 4.5: Security alerts configuration.

Figure 4.6: Device detailed parameters.

Figure 4.7: Device registration list.

#### 4.3.2 Web Administration Console (Desktop HMI)

The Web Admin Console provides system administrators with oversight across all deployed units:

- Telemetry Grid Overview: A real-time table logging data from all edge devices.

- Historical Logs Grid View: Searchable audit logs that allow administrators to inspect long-term temperature trends.

- Threshold Settings Panel: Form allowing admins to modify threshold limits.

Figure 4.8: Desktop Web Admin Dashboard overview.

Figure 4.9: Telemetry Grid overview logs.

Figure 4.10: Telemetry historical audits table.

Figure 4.11: System detailed parameters view.

Figure 4.12: Database threshold configurations control panel.

### 4.4 Code Fragments from the Codebase

#### 4.4.1 C++ Edge Caching Implementation (firmware/src/main.cpp)

The local buffering logic on the ESP32 handles connection drops using a local ring buffer struct:

struct TelemetryPoint {

float temp_c;

time_t timestamp;

};

TelemetryPoint telemetryBuffer[MAX_BUFFER_SIZE];

int bufferIndex = 0;

bool bufferFull = false;

void readTemperature() {

sensors.requestTemperatures();

float temp = sensors.getTempCByIndex(0);

if (temp == DEVICE_DISCONNECTED_C) {

Serial.println("Error: Sensor disconnected");

return;

}

Serial.printf("Temperature: %.2f°C\n", temp);

if (!client.connected()) {

telemetryBuffer[bufferIndex].temp_c = temp;

time_t now;

time(&now);

telemetryBuffer[bufferIndex].timestamp = now;

bufferIndex = (bufferIndex + 1) % MAX_BUFFER_SIZE;

if (bufferIndex == 0) bufferFull = true;

Serial.printf("Buffered reading (count: %d)\n",

bufferFull ? MAX_BUFFER_SIZE : bufferIndex);

}

}

#### 4.4.2 FastAPI Asynchronous WebSockets Broadcast (services/api/app/routers/telemetry.py)

The WebSocket connection manager enables real-time updates by managing active WebSocket connections:

class ConnectionManager:

def __init__(self):

self.active_connections: dict[str, list[WebSocket]] = {}

async def connect(self, websocket: WebSocket, device_id: str):

await websocket.accept()

if device_id not in self.active_connections:

self.active_connections[device_id] = []

self.active_connections[device_id].append(websocket)

def disconnect(self, websocket: WebSocket, device_id: str):

if device_id in self.active_connections:

self.active_connections[device_id].remove(websocket)

if not self.active_connections[device_id]:

del self.active_connections[device_id]

async def broadcast_to_device(self, device_id: str, message: dict):

if device_id in self.active_connections:

for connection in self.active_connections[device_id]:

try:

await connection.send_json(message)

except Exception:

pass

### 4.5 Experimental Testing Setup and Benchmarking Results

#### 4.5.1 Sensor Accuracy and Drift Calibrations

To calibrate the DS18B20 digital sensor, we evaluated its readings against a mercury reference thermometer. Measurements were taken at 5-minute intervals inside an incubation chamber.

#### Table 4.1: Temperature Calibration Data

| Interval | Reference Temp (°C) | DS18B20 Temp (°C) | Absolute Error (°C) |
| --- | --- | --- | --- |
| 1 | 37.00 | 36.84 | 0.16 |
| 2 | 37.30 | 37.12 | 0.18 |
| 3 | 37.50 | 37.31 | 0.19 |
| 4 | 37.80 | 37.54 | 0.26 |
| 5 | 38.00 | 37.71 | 0.29 |
| 6 | 38.30 | 38.06 | 0.24 |
| 7 | 38.50 | 38.22 | 0.28 |
| 8 | 38.80 | 38.54 | 0.26 |
| 9 | 39.00 | 38.72 | 0.28 |
| 10 | 39.20 | 38.94 | 0.26 |

The resulting Mean Absolute Error (MAE) is computed as follows:

MAE = 0.24°C

An MAE of 0.24°C is well within the acceptable ± 0.5°C limit, confirming the system's accuracy for poultry egg incubation (Trust, 2026).

#### 4.5.2 Real-time Alert Latency Distribution

We measured the latency for alert delivery under different network conditions. End-to-end latency is defined as the time from sensor reading to mobile UI update.

#### Table 4.2: Alert Latency Benchmarks

| Network Interface | Edge to Broker Latency (ms) | FastAPI Processing (ms) | FCM Notification Latency (ms) | Total Delay (ms) |
| --- | --- | --- | --- | --- |
| Wi-Fi (Farm LAN) | 88 | 12 | 1042 | 1142 |
| Mobile 4G Link | 154 | 14 | 1244 | 1412 |
| Mobile 3G Link | 344 | 22 | 1845 | 2211 |

Alerts delivered over Wi-Fi and 4G arrived well within the 2.0-second threshold. While 3G exceeded this limit slightly (2.21s), it remains acceptable in low-connectivity areas.

#### 4.5.3 Network Outage and Ring Buffer Recovery

To evaluate the offline caching buffer, we simulated Wi-Fi outages of varying durations.

#### Table 4.3: Outage Caching Recovery Rates

| Outage Duration (s) | Sampling Interval (s) | Data Packets Sampled | Buffer Packets Recovered | Recovery Rate (%) |
| --- | --- | --- | --- | --- |
| 10 | 5 | 2 | 2 | 100.0 |
| 30 | 5 | 6 | 6 | 100.0 |
| 60 | 5 | 12 | 12 | 100.0 |
| 120 | 5 | 24 | 20 | 83.3 |

For outages up to 100 seconds, the system achieved a 100% recovery rate. Once the outage exceeded 100 seconds (surpassing the 20-record buffer limit), the FIFO buffer began overwriting the oldest records as expected. This behavior preserved the most recent 20 readings, preventing data gaps once connection returned (Pacheco da Costa et al., 2023).

### 4.6 Security Update and Verification Results

We verified the firmware update alerts by flashing the ESP32 with an outdated version string (v1.0.1) while the PostgreSQL database specified v1.0.2 as required.

- Result: The FastAPI backend detected the version mismatch and sent a WebSocket alert payload to the mobile app in 120ms.

- UI Alert: A firmware mismatch warning banner was immediately rendered at the top of the mobile screen. Additionally, the backend pushed a version warning notification to the system tray via FCM.

### 4.7 User Acceptance Testing (UAT) and SUS Results

We evaluated the mobile interface with 10 poultry operators using the System Usability Scale (SUS) (Brooke, 1996).

#### Table 4.4: UAT SUS Score Calculations

| Operator ID | Q1 | Q2 | Q3 | Q4 | Q5 | Q6 | Q7 | Q8 | Q9 | Q10 | Scaled Score |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 1 | 4 | 2 | 4 | 1 | 5 | 2 | 4 | 1 | 90.0 |
| 2 | 4 | 2 | 4 | 2 | 3 | 2 | 4 | 2 | 3 | 2 | 70.0 |
| 3 | 5 | 2 | 5 | 1 | 4 | 1 | 5 | 1 | 4 | 1 | 92.5 |
| 4 | 4 | 1 | 3 | 2 | 4 | 2 | 4 | 2 | 3 | 1 | 75.0 |
| 5 | 3 | 2 | 4 | 3 | 3 | 2 | 3 | 3 | 4 | 2 | 62.5 |
| 6 | 4 | 2 | 4 | 1 | 4 | 2 | 4 | 1 | 4 | 2 | 80.0 |
| 7 | 5 | 1 | 5 | 2 | 4 | 1 | 5 | 2 | 4 | 1 | 90.0 |
| 8 | 4 | 3 | 3 | 3 | 3 | 2 | 4 | 2 | 3 | 2 | 65.0 |
| 9 | 3 | 2 | 4 | 2 | 4 | 2 | 3 | 2 | 4 | 1 | 72.5 |
| 10 | 4 | 1 | 4 | 2 | 3 | 1 | 4 | 2 | 3 | 1 | 77.5 |

The resulting Average SUS Usability Score is computed as follows:

Average SUS Score = 77.5 / 100

An average SUS score of 77.5 indicates "excellent" usability, confirming the interface is intuitive and ready for field operations.

### 4.8 Ethical Considerations

#### 4.8.1 User Privacy & Data Protection

To protect user privacy, the database stores only the operator's email address and password. No personal identifiers or GPS locations are logged.

#### 4.8.2 Access Control & Password Security

API access is restricted to authenticated operators using JWT signatures (Lengkong et al., 2025). Passwords are encrypted on the database using Bcrypt, protecting credentials against database compromises.

#### 4.8.3 Transit Confidentiality

Telemetry payloads are transmitted over encrypted transport channels (MQTTS/TLS port 8883) to prevent eavesdropping and interception (Verma & Ranga, 2018).

### 4.9 Summary

This chapter presented the implementation details of the Egg Guardian system. It described the development environments, HMI interfaces, and physical configurations. We evaluated system performance through calibration trials, latency distributions, network buffers, security check verifications, and UAT usability surveys. The results demonstrate the system is accurate, reliable, and ready for deployment. The next chapter summarizes the findings and presents recommendations for future work.

## CHAPTER 5: SUMMARY, CONCLUSION, & RECOMMENDATIONS

### 5.1 Summary of Findings

This study designed, implemented, and evaluated the Egg Guardian system—a secure, real-time cyber-physical monitoring solution for poultry egg storage and temperature regulation. The findings from the empirical evaluations in Chapter 4 are summarized as follows:

- Sensor Accuracy & Calibration: Calibration of the DS18B20 digital sensor against a mercury reference thermometer yielded a Mean Absolute Error (MAE) of 0.24°C, which is well within the acceptable ± 0.5°C threshold required for poultry egg incubation (Trust, 2026).

- Telemetry Latency: End-to-end data propagation latency averaged 1.14 seconds over a local farm Wi-Fi network and 1.41 seconds over a mobile 4G link. This meets the real-time operational delivery requirements (Nalendra & Waspada, 2025).

- Edge Resiliency & Recovery: The local FIFO caching buffer on the ESP32 achieved a 100% data recovery rate for simulated network dropouts lasting up to 100 seconds (Pacheco da Costa et al., 2023). Under longer outages, the ring buffer correctly overwrote the oldest telemetry points first, preserving the 20 most recent readings.

- Security Update Warnings: The system verified firmware versions in 120ms. When a version mismatch was detected, the backend successfully triggered warning badges in the mobile app and sent notifications via FCM (Dr. Brindha S et al., 2025).

- System Usability: Field testing with 10 poultry operators using the System Usability Scale (SUS) yielded an average usability score of 77.5 / 100, corresponding to "excellent" usability.

### 5.2 Final Conclusion

The Egg Guardian project demonstrates that low-cost, off-the-shelf microcontrollers (such as the ESP32) can manage poultry egg storage climates when paired with modern asynchronous protocols (MQTTS, WebSockets) and local caching queues (Protopappas et al., 2025). The system maintains telemetry accuracy and alerts operators to biological temperature boundary violations (37.0°C to 39.0°C) (FAO, 2024). Additionally, the version check handshake protects the edge network against outdated firmware vulnerabilities. In conclusion, the system provides a secure, reliable, and cost-effective monitoring solution for smart agricultural applications.

### 5.3 Practical and Commercial Implementation Issues

Deploying the system in commercial poultry hatcheries presents several practical challenges:

- Environmental Protection: Hatcheries are high-humidity, dust-prone environments. The ESP32 edge node must be mounted inside an IP66-rated casing, and the DS18B20 temperature probe must be housed in a waterproof stainless-steel sheath to prevent moisture damage and short circuits.

- Network Range Constraints: Metallic structures and thick concrete walls in commercial farms block Wi-Fi signals. Deploying the system across large facilities requires physical Wi-Fi repeaters or range extenders to ensure reliable connectivity.

- Power Redundancy: Commercial operations require continuous monitoring. The edge nodes should be integrated with an Uninterruptible Power Supply (UPS) or backup batteries to maintain operations during power outages.

### 5.4 Research Contributions to Agricultural Computing

This study contributes to the field of smart farming in two main areas:

- Data Integrity on the Edge: The study demonstrates that local caching queues allow low-cost microcontrollers to maintain data integrity during network drops (Pacheco da Costa et al., 2023). This reduces the need for expensive, high-RAM gateway hardware.

- Firmware Security Lifecycle Integration: The study integrates security update alerts directly into the agricultural monitoring loop (Dr. Brindha S et al., 2025). This addresses a common vulnerability in IoT deployments, where microcontrollers are left running outdated, insecure firmware.

### 5.5 Project Limitations

The current implementation has three main limitations:

- Wi-Fi Dependency: The ESP32 is restricted to 2.4GHz Wi-Fi. This limits system deployment in remote rural farms that lack local area network infrastructure.

- Memory Constraints: Due to the ESP32's limited RAM, the local ring buffer holds a maximum of 20 telemetry records. During outages longer than 100 seconds, the buffer begins overwriting historical data.

- FCM Gateway Dependency: The mobile notification system relies on Google's Firebase Cloud Messaging gateway. If Google services are offline or blocked, system tray alert notifications will fail to deliver.

### 5.6 Recommendations for Future Research

To build on this work, we recommend three directions for future research:

- LoRaWAN Integration: Replacing Wi-Fi with LoRaWAN to allow long-distance transmission (up to 15 km) in remote areas without internet access.

- Predictive Thermal Analysis: Deploying machine learning models (such as LSTM networks) on the backend server to analyze temperature trends and forecast heating/cooling failures before they occur.

- Self-Healing OTA Updates: Upgrading the update system from a warning banner to an automated Over-the-Air (OTA) patching pipeline that automatically updates the edge microcontroller when a version mismatch is detected.

Figure 5.1: Technical Future Research Roadmap block diagram, showing LoRaWAN gateways, LSTM predictive analysis, and secure self-healing OTA updates loops.

### 5.7 Summary

This chapter summarized the findings of the Egg Guardian system, presented conclusions, and discussed practical implementation challenges, research contributions, and limitations. Finally, it recommended future research paths, including LoRaWAN integration, predictive machine learning, and self-healing OTA updates.